EECA 2024 EnMS STARTER PACK

C7 · DATA COLLECTION PLAN

Energy Data Collection Plan

What to collect, from where, how often, and who owns it — companion to the 12-Month Consumption Tracker (xlsx)

GP46 para. 5.2.4(b) defines four data streams for the EnMS: the equipment and process inventory, energy consumption data (at least twelve consecutive months, utility bills at minimum), on-site generation data, and the variable data that drives consumption. This plan assigns each stream a source, frequency and owner; the accompanying xlsx tracker is where the numbers land.

| DOCUMENT NO. INV-ENMS-C7 | REV 1.0 | ISSUE DATE 29 July 2026 | PREPARED BY Innovast Sdn Bhd | APPROVED BY Anslem Lim, CEO |
| --- | --- | --- | --- | --- |

Rev 1.0 · issued 29 July 2026 · regulatory sources last verified 18 July 2026.

DRAFT FOR ADAPTATION — this template is not regulatory advice. Prepared with reference to GP/ST/No.46/2024 (Guidelines on Energy Management System), the Energy Efficiency and Conservation Act 2024 [Act 861] and the Energy Efficiency and Conservation Regulations 2024 [P.U.(A) 466/2024]; sources last verified 18 July 2026. Adapt every field to your organisation before use. Only Suruhanjaya Tenaga can determine compliance with the Act.

1	The four data streams (GP46 para. 5.2.4(b))

| Stream | What | Minimum standard | Lands in |
| --- | --- | --- | --- |
| 1 · Inventory | Operation systems, mechanical and electrical services, production processes and equipment | One controlled list, reviewed annually | Tracker — "SEU & Inventory" sheet |
| 2 · Consumption | All energy and energy resources purchased from suppliers (First and Second Schedule of the Act), in the billed units | ≥12 consecutive months of the most recent data, from utility bills; submetering for individual SEUs where available | Tracker — "Consumption" sheet |
| 3 · Generation | Energy generated on-site: electricity (kWh), chilled water (RTh), steam (tonnes), other forms | Monthly totals per generated stream | Tracker — "Generation" sheet |
| 4 · Variable data | Operating hours, production volume, weather (e.g. cooling degree days), occupancy — whatever moves your consumption | Monthly values, same periods as consumption | Tracker — "Variable Data" sheet |

2	Source map — fill in and keep current

| Data item | Source (account no. / meter / system) | Frequency | Owner | Cross-check |
| --- | --- | --- | --- | --- |
| Electricity (TNB or licensee) |  | Monthly bill |  | Meter reading vs bill |
| Natural gas / LPG / fuels |  | Monthly bill / delivery notes |  | Delivery quantity vs invoice |
| Purchased chilled water / steam (if any) |  | Monthly bill |  |  |
| On-site solar PV generation |  | Inverter / monitoring portal, monthly |  | Export vs import records |
| Other on-site generation (genset, boiler steam) |  | Log sheets, monthly totals |  | Fuel input records |
| Production output per product (industry) |  | ERP / production report, monthly |  | POE allocation basis |
| Operating hours / shifts / shutdown days |  | Operations calendar, monthly |  |  |
| Occupancy / rooms sold / beds / footfall (buildings) |  | FM or front-office system, monthly |  |  |
| GFA and area schedule (buildings) |  | As-built drawings — once, then on change |  | GP45 GFA exclusions applied |

3	Rules that keep the dataset defensible

- Billed units in, conversions referenced out. Record each fuel in the unit your supplier bills (kWh, GJ, tonnes, m³, MMBtu). The Commission’s online system converts to GJ automatically at reporting time (GP45 para. 4.1.5.2); for your own GJ totals, apply the conversion coefficients of GP/ST/No.44/2024 Appendix A, current revision — do not hand-copy coefficient tables into spreadsheets that will outlive the revision.

- Scheduled vs non-scheduled fuels. The Act’s definitions of energy and energy resources are the Second and First Schedules of Act 861 (electricity, chilled water, steam, hot water; natural gas, LPG constituents, coal, biomass, biogas, biodiesel, solar, etc.). Fuels outside the Schedules — diesel or fuel oil for gensets, for example — are not part of the statutory consumption totals: track them on a separate row for management purposes and keep them out of threshold and EE&C-report arithmetic.

- Twelve consecutive months, no gaps. A missing month breaks the baseline and the EE&C report dataset alike. If a bill is missing, chase the supplier copy — do not interpolate silently; if estimation is unavoidable, mark the cell and note the basis.

- The REM verifies against source documents. GP47 para. 3(e): reported data is compared with actual utility bills and invoices, and cross-examined against maintenance records, equipment specifications and audit reports. Keep the bill PDFs filed by month next to the tracker.

- SEU submetering is the upgrade path. Utility bills are the floor, not the ceiling. Where an SEU justifies it, submeter it — that is what turns POE from an audit estimate into measurement, and what InnoSense automates.

4	Ownership and cadence

| When | Who | Does |
| --- | --- | --- |
| Monthly, by the [10th] | Data owner(s) per Section 2 | Enter the month’s bills, generation and variable data into the tracker; file source documents |
| Monthly, by the [15th] | REM | Verify entries against bills; update EnPI/EEI; flag anomalies to the Committee |
| Quarterly (each Committee meeting) | Committee | Review consumption vs baseline and target; review data gaps and estimation flags |
| Annually | REM + Committee | Close the 12-month dataset for the EE&C report (C12); revisit this plan and the inventory |

This plan is a manual system. InnoSense replaces the monthly typing with metered data, continuous EnPI tracking and an always-current EE&C dataset. Register interest at eeca2024.my.

The complete Starter Pack

This document is one of thirteen working documents in the EECA 2024 EnMS Starter Pack — together they stand up an energy management system that follows GP/ST/No.46/2024 and the statutory duties of Act 861.

| Code | Document | Use it to |
| --- | --- | --- |
| C0 | Pack Guide & Compliance Clock | Orient the team; diarise the statutory deadlines |
| C1 | EECA EnMS Gap Assessment Checklist | Score your current position (Yes / Partial / No) against every GP46 element and statutory duty; get a readiness band |
| C2 | Energy Management Policy Template | Draft the policy with the four GP46-mandatory commitments built in |
| C3 | Committee & Governance Kit | Appoint the management representative; constitute the Committee; run and minute the meetings and the annual management review |
| C4 | Legal & Obligations Register | Hold every Act 861 / EECR duty with citation, deadline, penalty, owner and status — the legal-compliance evaluation GP46 para. 8.2(g) expects |
| C5 | REM Appointment Checklist | Choose internal vs external appointment; respect the tiering and the caps; manage the three-year window |
| C6 | EnPI / EEI Worksheet (sector-selectable) | Establish the baseline and indicator — SEC path for industry, BEI path for buildings |
| C7 | Energy Data Collection Plan + 12-Month Consumption Tracker (xlsx) | Plan and record the twelve months of consumption, generation and variable data GP46 requires (this document) |
| C8 | Action Plan & ESM Register (docx + xlsx) | Turn audit findings into a prioritised, costed, owned action plan with M&V per ESM |
| C9 | Communication, Awareness & Training Plan | Discharge GP46 para. 7.7–7.9 — communication matrix, motivation strategy, training programme |
| C10 | EnMS Self-Evaluation Matrix (xlsx) | Run the GP46 para. 5.2.3(a) maturity self-evaluation at Committee formation and periodically after |
| C11 | Work Procedure & Log Sheet Starters | Seed the operational controls — chiller plant and lighting examples to adapt |
| C12 | EE&C Report Skeleton | Assemble the annual report content in the GP45 structure before entering it in the Commission’s online system |

Get the complete pack, free:  eeca2024.my/enms-pack   ·   anslem@innovast.asia   ·   +60 12 355 6214